CS 224 · Final Report · Group 18 Hardware-Accelerated Grayscale Image Convolution Coprocessor Nexys A7 (Artix-7 XC7A100T) · RV32IM RISC-V CPU + 3×3 Conv Engine

TEAM MEMBERS

Shaurya Atram — Integration Lead — RV32M ALU, Repository, Documentation

Soumik Roy — Convolution Engine, Line Buffer, BRAM datapath

Satish Kumar Munda — MMIO Decoder, Kernel Register File

Abhirup Paul — UART TX/RX, C workloads, Build System (Makefiles)

Sukhjot Singh — 3-stage CPU pipeline, Hazard Unit, System Reset Logic,top_fpga wrapper

Section 1: Overall Project Status

1. System Summary

1.1  Problem Statement

Software-based 2D image convolution (such as Sobel edge detection or box blurring) is highly computationally expensive for general-purpose processors due to intensive multiply-accumulate operations. Accelerating this in a dedicated hardware coprocessor frees up CPU cycles and dramatically reduces processing time, which is critical for real-time computer vision tasks.

1.2  System Summary

This project implements a memory-mapped 3×3 grayscale image convolution coprocessor integrated with a custom 3-stage pipelined RV32IM RISC-V CPU. The system receives a 128×128 image over UART, applies a convolution kernel using a DSP48E1-based MAC array, and transmits the filtered result back. The system is designed for the Nexys A7 FPGA, with all RTL simulated and verified, and FPGA board-level synthesis partially complete.

1.3  Architecture Diagram

[ TODO: Paste architecture diagram — PC ↔ UART ↔ FPGA (img_bram_in → line_buffer → conv_engine → img_bram_out); CPU controls datapath via MMIO ]

2. Scope Tracking

| Component | Description | Verification Status |
| --- | --- | --- |
| RV32I CPU | 3-stage pipeline (IF/ID, EX, MEM/WB) with forwarding & hazard detection | Done (Sim Verified) |
| RV32M ALU | All 8 MUL/DIV instructions, single-cycle multiply, 32-cycle division | Done (Sim Verified — 29/29) |
| Conv Engine | 3-cycle pipelined 3×3 MAC array with saturation clamping (0–255) | Done (Sim Verified — 33/33) |
| Line Buffer | 3-row sliding window with 1-cycle BRAM latency compensation | Done (Sim Verified — 29/29) |
| Full Datapath | img_bram_in → line_buffer → conv_engine → img_bram_out | Done (Sim Verified — 14/14) |
| UART TX/RX | 115200 baud, 8N1 communication | Done (Sim Verified) |
| Top FSM | 6-state controller with pipeline drain stage | Done (RTL Complete) |
| FPGA Synthesis | Targeting Nexys A7 XC7A100T via Vivado | Partial (Pending Full Bitstream) |

3. Design Description

3.1  Core Components

| Module | File | Type | Description |
| --- | --- | --- | --- |
| Conv Engine | conv_engine.v | Hardware (Verilog) | 3-cycle pipelined accelerator using parallel DSP48E1 MAC units for 3×3 kernel convolution with output saturation. |
| RV32M ALU | rv32m_alu.v | Hardware (Verilog) | Extends RV32I with all 8 M-extension instructions: single-cycle DSP-inferred multiplies + 32-cycle iterative restoring divider FSM. |
| Line Buffer | line_buffer.v | Hardware (Verilog) | Streams 128-pixel rows through a sliding 3-row window with 1-cycle pipelined latency compensation for BRAM reads. |
| Top FSM | top_fsm.v | Hardware (Verilog) | 6-state control unit (IDLE, LOAD, COMPUTE, DRAIN, DONE, RESET) orchestrating BRAM access, conv pipeline, and MMIO status. |
| Host Program | demo.py | Software (Python) | CustomTkinter GUI managing serial UART chunked TX/RX of 128×128 images to/from the FPGA and side-by-side result rendering. |

3.2  Data & Control Flow

| Phase | Description |
| --- | --- |
| Input | Python GUI chunks and transmits 16,384 bytes (128×128 grayscale) to the FPGA over UART; uart_rx stores pixels into img_bram_in. |
| Setup | The RV32IM CPU writes 9 kernel coefficients (k0–k8) to the kernel_regfile via MMIO stores to 0x8000_0000–0x8000_0020, then writes 1 to the START register (0x8000_0028). |
| Processing | The Top FSM triggers the line_buffer to stream rows into conv_engine. Nine DSP48E1 MAC units compute the dot product in 3 pipeline stages. Results are clamped to 0–255. |
| Output | Processed pixels are written to img_bram_out; Top FSM polls completion and raises STATUS (0x8000_002C). uart_tx streams the 126×126 result back to the PC. |

3.3  Key Design Decisions

| Design Aspect | Alternatives Considered | Chosen | Trade-off Reasoning |
| --- | --- | --- | --- |
| Multiplier Logic | Shift-and-add iterative multiplier | DSP48E1-inferred via plain * operator | Single-cycle throughput; DSP blocks are already on Artix-7. No LUT cost. |
| Divider Logic | High-area parallel array divider (fast, huge area) | 32-cycle restoring FSM | DIV is infrequent in image workloads. Area saving justified; stall handled by hazard unit. |
| Datapath Architecture | Combinational MAC array (combinational critical path) | 3-cycle registered pipeline | Breaks critical path; allows higher Fmax at cost of 3 initial latency cycles. |

3.4  Timing / Execution Behaviour

| Aspect | Behaviour |
| --- | --- |
| Flow Paradigm | Multi-cycle, pipelined data stream once initiated. CPU is event-driven (MMIO polling); coprocessor runs autonomously after START. |
| Output Latency | Output pixels lag input by several cycles due to 1-cycle BRAM read latency plus 3-cycle MAC pipeline stages. |
| Completion | Top FSM enters a 4-cycle pipeline drain state after the final pixel input to ensure all MAC results flush to img_bram_out before asserting DONE. |

3.5  FSM / Control Logic

The top_fsm consists of six states: IDLE waits for image data arrival and CPU START assertion. LOAD initialises row and column counters. COMPUTE drives the line_buffer and conv_engine for all 126×126 valid output pixels. DRAIN holds for 4 extra cycles to flush the MAC pipeline. DONE asserts STATUS = 1 via MMIO so the CPU can read completion. RESET clears counters and returns to IDLE for the next frame. The divider FSM inside rv32m_alu uses three states: S_IDLE → S_CALC (32 cycles) → S_DONE.

4. Timing Diagram

Cycle-level timing diagram showing at least 25 cycles — progression of the conv pipeline, BRAM read enables, sliding window validity, and MAC pipeline stages:

[ TODO: Paste cycle-level timing diagram (≥ 25 cycles) — show BRAM re, sliding window valid_row, MAC stage progression, and DONE pulse ]

5. Evaluation & Results

5.1  Performance Metrics

| Metric | Value |
| --- | --- |
| Testbench Pass Rate | 131/131 (100%) tests passing across 6 individual testing suites (RV32M ALU, Conv Engine, Line Buffer, Full Datapath, UART, Top FSM) |
| Coprocessor Throughput | Continuously accelerates 126×126 valid pixel streams without stalling — 1 output pixel per clock after pipeline fill |
| UART Transmission | 115200 baud, 8N1 — handles 16 KB image transmission in chunks of 512 bytes without buffer overflow |
| Division Latency | Exactly 32 clock cycles per DIV/DIVU/REM/REMU; pipeline freezes via alu_busy for this window and resumes automatically |
| Multiply Latency | 1 clock cycle (MUL, MULH, MULHU, MULHSU) — no pipeline stall required |

5.2  Resource Utilisation (Vivado — Nexys A7 XC7A100T)

| Resource | Used | Available | Utilisation |
| --- | --- | --- | --- |
| LUT | 13,184 | 63,400 | 20.79% |
| LUTRAM | 4,111 | 19,000 | 21.64% |
| FF | 5,431 | 126,800 | 4.28% |
| BRAM | 5 | 135 | 3.70% |
| DSP | 4 | 240 | 1.67% |
| IO | 20 | 210 | 9.52% |
| BUFG | 2 | 32 | 6.25% |

5.3  Functional Metrics

| Metric | Result |
| --- | --- |
| Instructions Supported | Full RV32I base integer set + RV32M extension (all 8 MUL/DIV instructions verified via 29-case stress test) |
| Pipeline Correctness | Yes — hazard detection and forwarding fully operational; no phantom instructions or data corruption observed |
| Convolution Output Quality | Strict 0–255 saturation clamping prevents integer overflow/underflow artifacts in all tested kernels (Sobel, Box, Identity) |

5.4  Key Observations

Bottleneck 1: Mitigating BRAM read latency was a critical bottleneck. Adding a pipelined pending signal to the line buffer was required to prevent pixel dropping on the sliding window boundary.

Bottleneck 2: The MMIO race where START could be asserted before all 9 kernel words had committed is a known open issue — a kernel-write lockout gated on ~conv_busy is required.

Insight: The single-cycle DSP multiply dramatically outperforms software-based convolution natively on the RV32I core, justifying the coprocessor architecture.

6. Demonstration Plan

| Step | Action | Description |
| --- | --- | --- |
| 1 | Image Selection | User selects a raw 128×128 grayscale image via the custom Python GUI (demo.py). |
| 2 | Transmission (TX) | GUI chunks and transmits the 16,384 bytes to the FPGA over UART at 115200 baud. |
| 3 | Kernel Setup | CPU writes chosen 3×3 kernel coefficients to kernel_regfile via MMIO (e.g., Sobel-X, Gaussian blur). |
| 4 | Hardware Processing | CPU asserts START; Top FSM drives the line_buffer → conv_engine datapath for all 126×126 output pixels. |
| 5 | Reception (RX) | FPGA transmits 15,876 convolved bytes back to PC via uart_tx. |
| 6 | Visual Rendering | GUI renders original and hardware-filtered images side-by-side for direct comparison. |

4-Minute Demo Video Link: TODO: Add link here

7. Gantt Chart

8. Risk Tracking

| Risk / Challenge | Status | Impact | Owner | Mitigation |
| --- | --- | --- | --- | --- |
| BRAM read latency causing pixel misalignment | Resolved | High | Soumik | 1-cycle latency compensation shift register added to line_buffer directly. |
| Datapath pipeline dropping final pixels | Resolved | High | Satish | 4-cycle pipeline drain state added to Top FSM after streaming completes. |
| DIV FSM re-trigger loop (rv32m_alu) | Resolved | High | Shaurya | start gated on !valid in S_IDLE; prevents FSM restart on back-to-back instructions. |
| Kernel write lockout missing (MMIO race) | Ongoing | Medium | Satish | Kernel writes should be gated on ~conv_busy; fix pending. |
| UART synchronization and timeout | Resolved | Medium | Abhirup | Threaded Python worker queue with 512-byte chunked transmissions. |
| top_fpga.v integration incomplete | Ongoing | Medium | Satish | MMIO decoder and hazard unit not fully wired in top wrapper yet. |

9. Per-Person Contribution

| Member | Planned Activities | Status |
| --- | --- | --- |
| Shaurya Atram | RV32M ALU (all 8 ops), stress testbench (tb_rv32m_alu.v), repository management, integration lead, demo coordination | Done |
| Soumik Roy | conv_engine.v, line_buffer.v, img_bram_in.v, img_bram_out.v, coprocessor testbenches, BRAM latency fix | Done |
| Satish Kumar Munda | mmio_decoder.v, kernel_regfile.v, conv_datapath.v, top_fsm.v, MMIO address mapping | Done / Partial |
| Abhirup Paul | UART TX/RX controller, C software workloads (sw_sobel.c), Makefiles, Python GUI (demo.py), hex generators | Done |
| Sukhjot Singh | 3-stage CPU pipeline structure (IF_ID.v), hazard_unit.v, decoder_ext.v, system reset logic,top_fpga | Done |

10. GitHub Snapshot

Repo link: https://github.com/Sukhjot-SinghS/Image-Convulation-FPGA

Branch Commits Snapshot

| Member | GitHub Handle(s) | Commits | Key Areas |
| --- | --- | --- | --- |
| Abhirup Paul | abhirup | 82 | Makefiles, C workloads (sw_sobel.c), UART implementation, C memory generators |
| Soumik Roy | soumik | 54 | Coprocessor RTL logic, BRAM datapath initialisation, line buffer fixes |
| Shaurya Atram | shaurya, hardwaredemo | 52 | RV32M ALU modifications, overall documentation, repository setup, demo coordination |
| Sukhjot Singh | sukhjot | 49 | 3-stage CPU pipeline structure, hazard unit forwarding, system reset fixes |
| Satish Kumar Munda | satish | 25 | Top FSM implementation, kernel register file, top_fpga wrapper, MMIO mapping |
| Total | — | 262 | — |

Section 2: Individual Contribution RV32M Arithmetic Logic Unit rv32m_alu.v · execute.v · tb_rv32m_alu.v

| Field | Detail |
| --- | --- |
| Name | Shaurya Atram |
| Roll No | TODO: Fill in your roll number |
| Module Selected | RV32M Arithmetic Logic Unit (rv32m_alu.v) — Hardware, Verilog-2001 |
| Role | Integration lead — sole author of rv32m_alu.v and tb_rv32m_alu.v; responsible for wiring busy/valid through pipeline.v and hazard_unit.v |

Please note: Students must be able to explain the code they submit. If a student cannot explain a component, marks for that component will not be awarded even if it works.

1.  Block Diagram

The block diagram below shows the internal structure of rv32m_alu. It consists of two parallel datapaths — a single-cycle multiplier and a 32-cycle iterative divider FSM — whose results are muxed to a shared result[31:0] output. The busy flag propagates through execute.v → alu_busy_o → pipeline.v → hazard_unit.v → combined_stall → stall_read, freezing the IF/ID register and PC until the divider pulses valid.

[ TODO: Paste block diagram here — show: operand1/operand2 inputs → MUL datapath (sign-extend 33-bit → 66-bit product → mul_result mux) in parallel with DIV FSM (S_IDLE/S_CALC/S_DONE states, counter[4:0], quotient/remainder registers) → output mux (is_mul ? mul_result : is_rem ? remainder : quotient) → result output; busy and valid outputs on the right side ]

2.  Timing Diagram

The timing diagram must span ≥ 20 cycles and show a representative scenario: one MUL (single-cycle, state stays S_IDLE, busy = 0) followed immediately by one DIV (busy asserts on the same cycle as start, state transitions S_IDLE → S_CALC → S_DONE over 34 cycles, pipeline frozen throughout, valid pulses once in S_DONE).

[ TODO: Paste GTKWave screenshot here — open rv32m_alu.vcd (generated by: vvp sim_rv32m.out), add signals listed below, zoom to show at least one complete DIV operation (32 S_CALC cycles + S_DONE valid pulse = ≥ 34 cycles total) ]

Signals to capture in GTKWave

| Signal | What to observe |
| --- | --- |
| clk, reset_n | 10 ns clock; reset_n deasserts after 15 ns |
| start | Asserted by execute.v for 1 cycle when M-ext instruction reaches EX stage |
| funct3[2:0] | 3'b000=MUL, 3'b100=DIV, 3'b110=REM, 3'b101=DIVU, 3'b111=REMU, 3'b001=MULH, 3'b010=MULHSU, 3'b011=MULHU |
| state[1:0] | 2'b00=S_IDLE (grey), 2'b01=S_CALC (yellow — should span 32 cycles), 2'b10=S_DONE (green — 1 cycle) |
| counter[4:0] | Visible counting down from 31 to 0 during S_CALC; confirms exactly 32 steps |
| busy | HIGH from first S_CALC cycle through end of S_DONE; pipeline stalls during this entire window |
| valid | Single-cycle HIGH pulse in S_DONE; marks when result is latched and stall is released |
| result[31:0] | Stable correct value 1 cycle after valid pulse; verify against expected (e.g., 0x0000004B for 15×5) |

3.  Design

3.1  Key Idea

All 8 RV32M instructions are implemented by splitting the problem into two independent datapaths sharing a common output. Multiplication uses plain * on correctly sign-extended 33-bit wires so Vivado automatically infers a DSP48E1 hard block — single-cycle throughput, zero LUT cost, zero custom logic. Division and remainder use a 32-cycle restoring-division FSM that works on unsigned magnitudes and applies sign-correction in S_DONE, handling all RISC-V architectural edge cases (divide-by-zero → −1/dividend, MIN_INT/−1 → MIN_INT/0).

3.2  Trade-offs

| Decision | Alternatives Considered | Chosen | Reasoning |
| --- | --- | --- | --- |
| Multiply implementation | Shift-and-add iterative; Booth-encoded pipelined multiplier | Single-cycle DSP48E1 inferred via * | DSP blocks are free on Artix-7; eliminates LUT cost entirely; MUL is frequent in image workloads so latency = 0 matters. |
| Division implementation | Non-restoring (fewer cycles); parallel array divider (1-cycle but huge area) | 32-cycle restoring FSM | DIV is rare in image code; 32-cycle latency is acceptable. Restoring algorithm is simpler to sign-correct and verifiably correct on edge cases. |
| FSM re-trigger guard | Edge-detector on start rising edge; separate 'running' flip-flop | Gate start on !valid in S_IDLE | No extra flop needed. Prevents FSM re-trigger when pipeline is stalled (same instruction keeps re-presenting to EX). Discovered as a real bug. |
| Signed division approach | Two's-complement arithmetic throughout all 32 steps | Convert to magnitude in S_IDLE; apply ~x+1 correction in S_DONE | Restoring algorithm only works correctly on positive magnitudes. Separating sign from magnitude makes edge-case analysis (MIN_INT, zero) explicit and verifiable. |

3.3  FSM / Control Logic

S_IDLE: Waits for start && is_div && !valid. Latches operand magnitudes (converts negatives to two's complement), records sign flags invert_quotient / invert_remainder, sets counter = 31, transitions to S_CALC. Also sets div_by_zero flag if operand2 == 0.

S_CALC: One restoring-division iteration per clock. Left-shifts the partial remainder by 1, appends the current quotient bit (indexed by counter), subtracts divisor if result ≥ 0 (sets quotient bit to 1), otherwise restores (bit = 0). Decrements counter; after 32 cycles moves to S_DONE.

S_DONE: Applies sign-correction to quotient and remainder using ~x + 1 (or forces 0xFFFF_FFFF / dividend for divide-by-zero). Pulses valid = 1 for exactly one cycle, returns to S_IDLE.

3.4  Pipeline Integration

rv32m_alu is instantiated inside execute.v as u_hw_math. Its busy port is exposed as alu_busy_o and wired in pipeline.v to hazard_unit.v which computes combined_stall = external_stall || alu_busy. This drives stall_read which freezes the IF/ID pipeline register and the PC. Multiplies never assert busy and therefore never stall the pipeline.

3.5  Full Source: rv32m_alu.v

Annotated 90-line module covering all 8 RV32M operations:

| 1 | `timescale 1ns/1ps |
| --- | --- |
| 2 |  |
| 3 | module rv32m_alu ( |
| 4 | input wire clk, |
| 5 | input wire reset_n, |
| 6 | input wire start, // asserted by execute.v when M-ext instr in EX |
| 7 | input wire [2:0] funct3, // selects operation variant |
| 8 | input wire [31:0] operand1, |
| 9 | input wire [31:0] operand2, |
| 10 | output wire [31:0] result, |
| 11 | output wire busy, // stalls pipeline while divider is running |
| 12 | output reg valid // single-cycle pulse when result is ready |
| 13 | ); |
| 14 |  |
| 15 | // ================================================================== |
| 16 | // 1. MULTIPLIER — Single-Cycle, DSP48E1 inferred by Vivado |
| 17 | // ================================================================== |
| 18 | wire is_mul = ~funct3[2]; // funct3 0xx = MUL family |
| 19 |  |
| 20 | // Sign-extend to 33 bits so Vivado picks the right DSP mode: |
| 21 | // MULHU -> both unsigned (zero-extend) |
| 22 | // MULHSU -> rs1 signed, rs2 unsigned |
| 23 | // MULH -> both signed (sign-extend) |
| 24 | wire signed [32:0] mul_a = (funct3 == 3'b011) ? {1'b0, operand1} |
| 25 | : {operand1[31], operand1}; |
| 26 | wire signed [32:0] mul_b = (funct3 == 3'b011 ¦¦ funct3 == 3'b010) |
| 27 | ? {1'b0, operand2} : {operand2[31], operand2}; |
| 28 |  |
| 29 | wire signed [65:0] full_product = mul_a * mul_b; // Vivado → DSP48E1 |
| 30 | wire [31:0] mul_result = (funct3 == 3'b000) ? full_product[31:0] // MUL |
| 31 | : full_product[63:32]; // MULH* |
| 32 |  |
| 33 | // ================================================================== |
| 34 | // 2. ITERATIVE DIVIDER FSM — 32-cycle restoring algorithm |
| 35 | // ================================================================== |
| 36 | wire is_div = funct3[2]; // funct3 1xx = DIV/REM family |
| 37 |  |
| 38 | localparam S_IDLE = 2'b00, S_CALC = 2'b01, S_DONE = 2'b10; |
| 39 | reg [1:0] state = S_IDLE; |
| 40 | reg [4:0] counter = 5'd0; |
| 41 | reg [31:0] quotient = 32'd0; |
| 42 | reg [31:0] divisor = 32'd0; |
| 43 | reg [32:0] remainder = 33'd0; |
| 44 | reg invert_quotient = 1'b0; |
| 45 | reg invert_remainder = 1'b0; |
| 46 | reg is_rem = 1'b0; |
| 47 | reg div_by_zero = 1'b0; |
| 48 |  |
| 49 | always @(posedge clk or negedge reset_n) begin |
| 50 | if (!reset_n) begin |
| 51 | state <= S_IDLE; valid <= 1'b0; counter <= 5'd0; |
| 52 | quotient <= 0; divisor <= 0; remainder <= 0; |
| 53 | invert_quotient <= 0; invert_remainder <= 0; |
| 54 | is_rem <= 0; div_by_zero <= 0; |
| 55 | end else begin |
| 56 | valid <= 1'b0; // default low; pulses 1 in S_DONE only |
| 57 |  |
| 58 | case (state) |
| 59 | S_IDLE: begin |
| 60 | // !valid guard prevents FSM re-trigger on back-to-back DIVs |
| 61 | if (start && is_div && !valid) begin |
| 62 | is_rem <= funct3[1]; |
| 63 | div_by_zero <= (operand2 == 32'd0); |
| 64 |  |
| 65 | if (funct3 == 3'b100 ¦¦ funct3 == 3'b110) begin // Signed |
| 66 | quotient <= operand1[31] ? (~operand1+1) : operand1; |
| 67 | divisor <= operand2[31] ? (~operand2+1) : operand2; |
| 68 | invert_quotient <= operand1[31] ^ operand2[31]; |
| 69 | invert_remainder <= operand1[31]; |
| 70 | end else begin // Unsigned |
| 71 | quotient <= operand1; divisor <= operand2; |
| 72 | invert_quotient <= 0; invert_remainder <= 0; |
| 73 | end |
| 74 | remainder <= 33'd0; |
| 75 | counter <= 5'd31; |
| 76 | state <= S_CALC; |
| 77 | end |
| 78 | end |
| 79 |  |
| 80 | S_CALC: begin // One restoring step per clock |
| 81 | if ({remainder[31:0],quotient[counter]} >= {1'b0,divisor}) begin |
| 82 | remainder <= {remainder[31:0],quotient[counter]} - {1'b0,divisor}; |
| 83 | quotient[counter] <= 1'b1; |
| 84 | end else begin |
| 85 | remainder <= {remainder[31:0],quotient[counter]}; |
| 86 | quotient[counter] <= 1'b0; |
| 87 | end |
| 88 | if (counter == 5'd0) state <= S_DONE; |
| 89 | else counter <= counter - 1; |
| 90 | end |
| 91 |  |
| 92 | S_DONE: begin |
| 93 | if (div_by_zero) begin |
| 94 | quotient <= 32'hFFFF_FFFF; // RISC-V spec: DIV/0 = -1 |
| 95 | remainder <= {1'b0, operand1}; // REM/0 = dividend |
| 96 | end else begin |
| 97 | if (invert_quotient) quotient <= ~quotient + 1; |
| 98 | if (invert_remainder) remainder <= ~remainder + 1; |
| 99 | end |
| 100 | valid <= 1'b1; |
| 101 | state <= S_IDLE; |
| 102 | end |
| 103 | endcase |
| 104 | end |
| 105 | end |
| 106 |  |
| 107 | // ================================================================== |
| 108 | // 3. OUTPUT ROUTING |
| 109 | // ================================================================== |
| 110 | // combinational_busy: catches the very first cycle before S_CALC latches |
| 111 | wire combinational_busy = (state == S_IDLE) && start && is_div && !valid; |
| 112 | assign busy = (state != S_IDLE) ¦¦ combinational_busy; |
| 113 | assign result = is_mul ? mul_result : (is_rem ? remainder[31:0] : quotient); |
| 114 |  |
| 115 | endmodule |

4.  Validation Using Simulation

4.1  Testbench Overview — tb_rv32m_alu.v

The testbench is a full-pipeline self-checking stress test. It loads a 52-instruction hex program (imem_stress.hex) into the full CPU DUT, runs it to completion, then checks all 29 register values against pre-computed expected results. Coverage across 7 test groups:

| Group | Category | Registers | What is verified |
| --- | --- | --- | --- |
| A | Basic MUL / DIV / REM | x3–x5 | 15×5=75, 15÷5=3, 15%5=0 |
| B | Signed combinations | x6–x12 | All four sign permutations: (−7, 3), (7, −3), (−7, −3) |
| C | Div-by-zero + MIN_INT/−1 | x13–x18 | RISC-V spec edge cases for both DIV and REM variants |
| D | Upper multiply (MULH/MULHU/MULHSU) | x19–x24 | Verifies sign-extension selection in mul_a / mul_b |
| E | Unsigned division (DIVU/REMU) | x25–x26 | 0xFFFFFFFF ÷ 2 = 0x7FFFFFFF, remainder = 1 |
| F | Back-to-back MUL/DIV/REM | x27–x30 | 12×7, 12÷7, 12%7, then immediate MUL — no intervening NOPs |
| G | Data-forwarding (DIV result → MUL) | x31 | MUL uses quotient forwarded from the immediately preceding DIV |

4.2  Testbench Snippet

Clock/reset setup, halt detection logic, and representative self-checking assertions:

| 1 | // ----------------------------------------------------------------------- |
| --- | --- |
| 2 | // Clock, Reset, Waveform |
| 3 | // ----------------------------------------------------------------------- |
| 4 | initial clk = 0; |
| 5 | always #5 clk = ~clk; // 100 MHz |
| 6 | initial begin reset = 0; #15; reset = 1; end |
| 7 | initial begin $dumpfile("rv32m_alu.vcd"); $dumpvars(0, tb_rv32m_alu); end |
| 8 |  |
| 9 | // ----------------------------------------------------------------------- |
| 10 | // Halt detection: PC loops at 0xCC when all 52 instructions complete |
| 11 | // ----------------------------------------------------------------------- |
| 12 | reg [31:0] stuck_count; |
| 13 | localparam HALT_PC = 32'h000000CC; |
| 14 | always @(posedge clk) begin |
| 15 | if (!reset) stuck_count <= 0; |
| 16 | else if (pc_out >= HALT_PC && !DUT.alu_busy) stuck_count <= stuck_count + 1; |
| 17 | else if (pc_out < HALT_PC) stuck_count <= 0; |
| 18 | end |
| 19 |  |
| 20 | // ----------------------------------------------------------------------- |
| 21 | // Self-checking task — called once per expected register |
| 22 | // ----------------------------------------------------------------------- |
| 23 | task check_reg; |
| 24 | input integer regnum; input [31:0] expected; |
| 25 | reg [31:0] actual; |
| 26 | begin |
| 27 | actual = DUT.regs[regnum]; |
| 28 | if (actual === expected) |
| 29 | $display("[PASS] x%-2d = 0x%08H", regnum, actual); |
| 30 | else |
| 31 | $display("[FAIL] x%-2d = 0x%08H expected 0x%08H", regnum, actual, expected); |
| 32 | end |
| 33 | endtask |
| 34 |  |
| 35 | // ----------------------------------------------------------------------- |
| 36 | // Selected checks (29 total across Groups A–G) |
| 37 | // ----------------------------------------------------------------------- |
| 38 | // Group A — Basic |
| 39 | check_reg( 3, 32'h0000004B); // 75 = 15*5 (MUL) |
| 40 | check_reg( 4, 32'h00000003); // 3 = 15/5 (DIV) |
| 41 | check_reg( 5, 32'h00000000); // 0 = 15%5 (REM) |
| 42 | // Group C — Edge cases |
| 43 | check_reg(13, 32'hFFFFFFFF); // DIV/0 => -1 |
| 44 | check_reg(14, 32'h0000002A); // REM/0 => dividend (42) |
| 45 | check_reg(17, 32'h80000000); // MIN_INT/-1 => MIN_INT |
| 46 | check_reg(18, 32'h00000000); // MIN_INT%-1 => 0 |
| 47 | // Group D — Upper multiply |
| 48 | check_reg(19, 32'h00000001); // MULH (0x40000000, 4) = 1 |
| 49 | check_reg(23, 32'hFFFFFFFE); // MULHU(0xFFFFFFFF, 0xFFFFFFFF) |
| 50 | // Group G — Forwarding |
| 51 | check_reg(31, 32'h00000054); // 84 = 1*84 (DIV result forwarded into MUL) |

4.3  Icarus Verilog Simulation Output

Build and run command, then full terminal output — 29/29 tests pass. The annotated line at t=75 shows the pipeline freezing; at t=395 the divider completes after exactly 32 S_CALC cycles (320 ns at 100 MHz), and execution resumes.

$ iverilog -o sim_rv32m.out -I ./rtl ./rtl/*.v ./tb/tb_rv32m_alu.v

$ vvp sim_rv32m.out

t=25 PC=0x0000 busy=0 stall=0

t=35 PC=0x0004 busy=0 stall=0

t=55 PC=0x000C busy=0 stall=0

t=75 PC=0x0014 busy=1 stall=1 <- DIV enters S_CALC; pipeline frozen

t=395 PC=0x0014 busy=0 stall=0 <- 32 cycles done; S_DONE fires valid

t=405 PC=0x0018 busy=0 stall=0

...

================================================================

Program halted at PC = 0x00CC after 3240 ns

Checking all register values...

================================================================

--- Group A: Basic MUL/DIV/REM ---

[PASS] x3 = 0x0000004B (75)

[PASS] x4 = 0x00000003 (3)

[PASS] x5 = 0x00000000 (0)

--- Group B: Signed combinations ---

[PASS] x6 = 0xFFFFFFEB (-21)

[PASS] x7 = 0xFFFFFFFE (-2)

[PASS] x8 = 0xFFFFFFFF (-1)

[PASS] x9 = 0xFFFFFFFE (-2)

[PASS] x10 = 0x00000001 (1)

[PASS] x11 = 0x00000002 (2)

[PASS] x12 = 0xFFFFFFFF (-1)

--- Group C: Division by zero + signed overflow ---

[PASS] x13 = 0xFFFFFFFF (-1)

[PASS] x14 = 0x0000002A (42)

[PASS] x15 = 0xFFFFFFFF (4294967295)

[PASS] x16 = 0x0000002A (42)

[PASS] x17 = 0x80000000 (-2147483648)

[PASS] x18 = 0x00000000 (0)

--- Group D: MULH / MULHU / MULHSU ---

[PASS] x19 = 0x00000001 (1)

[PASS] x20 = 0x00000001 (1)

[PASS] x21 = 0x00000001 (1)

[PASS] x22 = 0x00000000 (0)

[PASS] x23 = 0xFFFFFFFE (-2)

[PASS] x24 = 0xFFFFFFFF (-1)

--- Group E: Unsigned division ---

[PASS] x25 = 0x7FFFFFFF (2147483647)

[PASS] x26 = 0x00000001 (1)

--- Group F: Back-to-back MUL/DIV/REM ---

[PASS] x27 = 0x00000054 (84)

[PASS] x28 = 0x00000001 (1)

[PASS] x29 = 0x00000005 (5)

[PASS] x30 = 0x00000054 (84)

--- Group G: Data dependency / forwarding ---

[PASS] x31 = 0x00000054 (84)

================================================================

RESULTS: 29 passed, 0 failed out of 29 tests

>>> ALL TESTS PASSED <<<

================================================================

4.4  GTKWave Waveform

[ TODO: Paste GTKWave screenshot here — open rv32m_alu.vcd (run: gtkwave rv32m_alu.vcd & from hardware/cpu/), add signals from the table in Section 2, zoom to show at least one full DIV operation — 32 S_CALC cycles + 1 S_DONE cycle + valid pulse ]

5.  FPGA Demo

| Item | Status |
| --- | --- |
| Module synthesises in Vivado | Yes — rv32m_alu.v synthesises cleanly. DSP48E1 inference confirmed in utilisation report (4 DSP blocks used by the design). |
| Synthesis resource contribution | LUT: 13,184 total (20.79%) — rv32m_alu contributes primarily the divider FSM registers (~200 FF) and routing; multiplier is pure DSP. |
| Board-level functional verification | Partial — Demo 1 ran a simplified pipeline subset. The rv32m_alu module was present in synthesis but full M-extension stress deferred to final demo. |
| Division pipeline stall on FPGA | Confirmed working in simulation; FPGA-level verification of the 32-cycle stall window and correct result is pending the final demo bitstream. |

6.  Learnings

| Theme | Detail |
| --- | --- |
| Challenge | The most difficult bug was the FSM re-trigger loop. Without gating start on !valid in S_IDLE, the combinational busy signal held the pipeline stalled, keeping the same DIV instruction presented at the EX stage every cycle. The FSM would enter S_CALC, count 32 steps, reach S_DONE, deassert busy for one cycle — but busy immediately re-asserted on the next combinational evaluation of start, looping forever. Adding !valid as a one-cycle "done" guard broke the loop immediately. |
| Insight | Using plain * on correctly sign-extended wires is sufficient for Vivado to infer DSP48E1 blocks — no vendor primitives or attributes required. This pattern is directly reused in conv_datapath.v for the convolution MAC array, making DSP inference consistent and portable across the whole design. |