Hardware-Accelerated Grayscale Image Convolution Coprocessor

Group 18  —  CS 224,  Nexys A7 (Artix-7 XC7A100T)

Project Design Status Report  |  April 2026

# 1.  System Summary

A hardware-accelerated 3×3 grayscale convolution coprocessor on Nexys A7 (Artix-7), driven by a custom RV32I + RV32M 3-stage pipeline with DSP48E1-inferred MAC units and a memory-mapped MMIO coprocessor interface. Integration stage — Phase 1 (RV32M 8-op extension) complete; Phase 2 coprocessor RTL simulation-verified; full FPGA board-level synthesis pending. RV32M ops, the conv engine, and line buffer are simulation-verified; MMIO decoder integrated. CustomTkinter GUI and C software workloads active.

# 2.  Scope Tracking

| # | Feature | Status | Target Date | Evidence |
| --- | --- | --- | --- | --- |
| MS1 | RV32M 8-op ALU (MUL, MULH, MULHSU, MULHU, DIV, DIVU, REM, REMU) via rv32m_alu with DSP48E1 inference and 32-cycle iterative divider FSM. | Done | Mar 31, 2026 | Testbench verified |
| MS2 | Pipeline stall, hazard unit connections, and decoder extension for RV32M instructions integrated inside the IF/ID stage. | Done | Apr 3, 2026 | Design / RTL verified |
| GS1 | 3×3 conv engine (conv_engine.v) with DSP48E1 MAC array — 9-tap dot product per output pixel. | Partial | Apr 18, 2026 | Testbench verified |
| GS2 | Line buffer (line_buffer.v) with pipelined BRAM — synchronous latency resolved via pipeline register insertion. | Partial | Apr 18, 2026 | Testbench verified |
| GS3 | MMIO decoder: kernel regs 0x80000000–0x80000020, START at 0x80000024, STATUS at 0x80000028, NORM at 0x80000030, SW_DONE at 0x80000034. | Partial | Apr 12, 2026 | Design: 2965104 |
| STR1 | Triple-Channel RGB Processing: Expand the datapath to handle full RGB color channels, utilizing 27 DSP48E1 blocks (~11% of available DSPs). | Not Started | — | — |
| STR2 | Interactive Kernel Switching: Enable live, interactive kernel switching by using on-board physical switches during the live FPGA demonstration. | Not Started | — | — |

# 3.  Design Goals Tracking

| Metric | Target | Current | Status |
| --- | --- | --- | --- |
| Latency | ≤ (W×H + 4) cycles | TBD — functional timing verification pending board bring-up. | TBD |
| Throughput | 1 pixel/cycle | 1 pixel/cycle (sim-verified in tb_conv_engine) | Met |
| LUT | ≤ 40,000 | 13,641 LUTs used (21.52% of 63,400) — Vivado synth Apr 19 2026 | Met |
| FF | ≤ 40,000 | 5,429 FFs used (4.28% of 126,800) — Vivado synth Apr 19 2026 | Met |
| BRAM | ≤ 8 | 5 RAMB36E1 used (3.70% of 135) — Vivado synth Apr 19 2026 | Met |
| DSP | ≤ 12 | 4 DSP48E1 used (1.67% of 240) — Vivado synth Apr 19 2026 | Met |
| Correctness | 100% SW reference match | Simulation tests passing | Met (simulation) |

# 4.  Demonstration Plan Status  (Apr 25–26)

| Stage | Planned | Current Status |
| --- | --- | --- |
| Input | Load 8-bit grayscale BMP via UART → BRAM controller → img_bram_in | Sim-verified; GUI sends image bytes over serial. |
| Processing | CPU → MMIO → conv engine → poll STATUS until done | MMIO integrated; sw_gaussian_blur.c runs on pipeline. |
| Output | UART TX → GUI display + save to PNG | CustomTkinter GUI ready; output save to PNG implemented. |

# 5.  Gantt Chart  (Mar 22 – Apr 26)

|  |  | March |  |  |  |  |  |  |  |  |  | April |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Owner | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| rv32m_alu.v + Testbench + GUI | Shaurya |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| hazard_unit.v + System Integration | Sukhjot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| conv_engine + line_buffer + img_bram | Soumik |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MMIO decoder + conv_datapath + top_fsm | Satish |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UART controller + C workloads | Abhirup |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 6.  Risk Tracking

| # | Risk | Status | Severity | Owner | Mitigation |
| --- | --- | --- | --- | --- | --- |
| 1 | DIV multi-cycle handling | Resolved | High | Shaurya | FSM gates start on ~div_busy; re-trigger loop fixed. |
| 2 | Kernel write lockout missing | Ongoing | Medium | Satish | Gate kernel reg writes on ~conv_busy in mmio_decoder; enforce all 9 writes before asserting START. |
| 3 | MMIO address mismatch (doc vs RTL) | New | Low | Satish / Shaurya | Doc lists START=0x80000028; RTL uses 0x80000024. Confirm final addresses before demo. |
| 4 | FPGA synthesis not complete | Ongoing | High | Shaurya | Run Vivado implementation before Apr 25 to get resource report and confirm timing closure. |
| 5 | Infinite software freeze during HW polling | New | High | Abhirup | Implement maximum cycle timeout in the C polling loop while(HW_DONE_REG == 0). |
| 6 | Output BRAM boundary index overflow | New | Low | Abhirup | Verify C-loop indices physically map to 0..15875 to prevent overwriting hardware MMIO registers. |
| 7 | Vivado stale cache / zombie .hex files | Resolved | High | Sukhjot | Bypass Vivado auto-import by hardcoding absolute file paths ($readmemh) directly in instr_mem.v. |
| 8 | Combinatorial loop in MMIO read path | Resolved | High | Sukhjot | Added 1-cycle latency flip-flop (is_mmio_read_wb) to memory mux to break loop. |

# 7.  Per-Person Contribution

| Member | Contribution | Status | Last Updated |
| --- | --- | --- | --- |
| Shaurya | rv32m_alu.v — all 8 RV32M ops with DSP48E1 inference and 32-cycle iterative divider FSM. tb_rv32m_alu.v. CustomTkinter Python GUI implementation. | Done / WIP | Apr 5, 2026 |
| Sukhjot | hazard_unit.v and base CPU upgrades. System integration (linking convolution engine to GUI/software) and bug fixing. | Done / WIP | Apr 19, 2026 |
| Soumik | conv_engine.v, line_buffer.v, img_bram_in/out.v — Phase 2 RTL. | WIP | Apr 12, 2026 |
| Satish | mmio_decoder.v and conv_datapath.v — Memory mapping and datapath wiring. top_fsm.v state logic. | WIP | Apr 25, 2026 |
| Abhirup | UART controller, bare-metal C benchmark codes (sw_blur.c). | WIP | Apr 19, 2026 |

# 8.  GitHub Snapshot

Repository: https://github.com/Sukhjot-SinghS/Image-Convulation-FPGA

Total commits: 103

## Commit Counts by Member

| Shaurya | Sukhjot | Soumik | Satish | Abhirup |
| --- | --- | --- | --- | --- |
| 12 | 31 | 6 | 27 | 27 |

## Key Commits

| Hash | Message | Description |
| --- | --- | --- |
| 8b22e70 | Integrate coprocessor, UART + SW utilities (Clean) | Full Phase 2 integration: fixed line_buffer BRAM latency, wired top_fsm drain, brought in UART controller and C workloads. |
| 2965104 | Full MMIO decoder for kernel regfile + status reads | Maps kernel weights to 0x80000000–0x80000020, START to 0x80000024, STATUS to 0x80000028; adds SW_DONE doorbell. |
| 87ff069 | Add CustomTkinter GUI for FPGA coprocessor | Premium dark-mode GUI: sends 128×128 image over UART, reads 126×126 convolved output, displays side-by-side comparison, saves convolved PNG. |
| 56357c8 | Add RV32M ALU module | Initial rv32m_alu.v: 8-op RV32M ALU with single-cycle multiply (DSP48E1) and 32-cycle iterative restoring divider FSM. |
| 903e18e | Add sw_sobel + rename to blur (Sukhjot) | Renamed sw_sobel.c → sw_gaussian_blur.c; updated Python GUI to save convolved output as timestamped PNG. |

# 9.  Changes Based on TA Inputs

| Feedback Item | Date Received | Action Taken | Status |
| --- | --- | --- | --- |