Section 2: Individual Contribution RV32M Arithmetic Logic Unit rv32m_alu.v · execute.v · tb_rv32m_alu.v

| Name | Shaurya Atram |
| --- | --- |
| Roll No | TODO: Fill in |
| Module | rv32m_alu.v (Hardware — Verilog-2001) |
| Role | Integration lead — owner of RV32M ALU and its stress-test bench |

1.  Block Diagram

The block diagram below shows the internal structure of rv32m_alu. It consists of two parallel datapaths — a single-cycle multiplier and a 32-cycle iterative divider FSM — whose results are muxed to a shared result output. The busy flag is fed back through execute.v → hazard_unit.v to stall the pipeline register (IF/ID and EX pipeline regs) during multi-cycle division.

[ TODO: Paste block diagram image here — show operand inputs, MUL datapath (DSP48E1), DIV FSM (S_IDLE/S_CALC/S_DONE), output mux, busy/valid outputs ]

2.  Timing Diagram

The timing diagram must cover ≥ 20 cycles and show at minimum: clk, reset_n, start, funct3, state, counter, busy, valid, result. A representative scenario is: one DIV (signed, 32 cycles in S_CALC) preceded by one MUL (single cycle, remains in S_IDLE). This illustrates the pipeline stall window clearly.

[ TODO: Paste GTKWave screenshot here (open rv32m_alu.vcd, add signals listed below, zoom to cover at least one full division operation ≥ 20 cycles) ]

Signals to include in GTKWave

| Signal | What to observe |
| --- | --- |
| clk, reset_n | Clock and active-low reset |
| start | Asserted by execute.v for one cycle when M-ext instr enters EX |
| funct3[2:0] | Selects operation: 0xx = MUL family, 1xx = DIV/REM family |
| state[1:0] | 00 = S_IDLE, 01 = S_CALC, 10 = S_DONE |
| counter[4:0] | Decrements 31→0 across S_CALC (32 cycles) |
| busy | HIGH throughout S_CALC + combinational in S_IDLE when start fires |
| valid | Single-cycle pulse in S_DONE; used by pipeline to release stall |
| result[31:0] | Carries final quotient/remainder/product after valid |

3.  Design

3.1  Key Idea

The module implements all 8 RV32M instructions by splitting the problem into two independent datapaths sharing a common output. Multiplication uses plain * on sign-extended 33-bit wires so that Vivado automatically infers a DSP48E1 hard block — giving single-cycle throughput with no custom logic. Division and remainder use a 32-cycle restoring-division FSM (S_IDLE → S_CALC → S_DONE) that works on unsigned magnitudes and corrects the sign in S_DONE, correctly handling all RISC-V edge cases (divide-by-zero, MIN_INT / −1).

3.2  Trade-offs

| Decision | Alternatives Considered | Reasoning / Trade-off |
| --- | --- | --- |
| Single-cycle multiply via DSP48E1 | Shift-and-add iterative; Booth-encoded pipeline | DSP block is already present on Artix-7; eliminates LUT cost entirely. MUL is common in vector/matrix workloads so single-cycle throughput matters. |
| 32-cycle restoring division | Non-restoring (fewer cycles); parallel array divider (fast, huge area) | DIV is infrequent in image-processing code; 32-cycle latency is acceptable. Restoring algorithm is simpler to sign-correct and avoids the large combinational fan-out of a parallel divider. |
| busy gated on !valid in S_IDLE | Edge-detector on start; separate 'running' flip-flop | Prevents FSM re-trigger on back-to-back instructions without a separate edge-detection flop. Discovered as a real bug: omitting !valid caused the FSM to loop and never finish. |
| Magnitude + sign-correction in S_DONE | Two's-complement arithmetic throughout FSM | Restoring division only works correctly on unsigned magnitudes. Converting to magnitude in S_IDLE and applying ~x+1 in S_DONE is cleaner and makes edge-case analysis (MIN_INT, zero) straightforward. |

3.3  FSM Description

The divider FSM has three states. S_IDLE: waits for start && is_div && !valid; latches operand magnitudes and sign flags, sets counter to 31, transitions to S_CALC. S_CALC: one restoring-division step per cycle — shifts the partial remainder left by 1 and appends the next quotient bit; decrements counter; after 32 cycles moves to S_DONE. S_DONE: applies sign-correction to quotient and remainder (or forces −1/dividend for divide-by-zero), pulses valid, returns to S_IDLE.

3.4  Integration with the Pipeline

The module sits inside execute.v as instance u_hw_math. Its busy output is exposed as alu_busy_o and wired through pipeline.v to hazard_unit.v, which OR's it with any external stall to produce combined_stall. This freezes both the IF/ID register and the PC until the divider asserts valid. Multiplies assert busy = 0 throughout and therefore never stall the pipeline.

3.5  Full Source: rv32m_alu.v

Complete annotated module — 123 lines covering all 8 RV32M operations.

| 1 | `timescale 1ns/1ps |
| --- | --- |
| 2 |  |
| 3 | module rv32m_alu ( |
| 4 | input wire clk, |
| 5 | input wire reset_n, |
| 6 | input wire start, // 1 when M-ext instr in EX stage |
| 7 | input wire [2:0] funct3, // selects MUL/MULH/DIV/REM variant |
| 8 | input wire [31:0] operand1, |
| 9 | input wire [31:0] operand2, |
| 10 | output wire [31:0] result, |
| 11 | output wire busy, // stalls pipeline when high |
| 12 | output reg valid // pulses 1 cycle when result ready |
| 13 | ); |
| 14 |  |
| 15 | // ================================================================== |
| 16 | // 1. MULTIPLIER (Single-Cycle, DSP48E1 inferred) |
| 17 | // ================================================================== |
| 18 | wire is_mul = ~funct3[2]; // funct3[2]=0 => MUL family |
| 19 |  |
| 20 | // Sign-extend operands: MULHU treats both as unsigned |
| 21 | wire signed [32:0] mul_a = (funct3 == 3'b011) ? {1'b0, operand1} |
| 22 | : {operand1[31], operand1}; |
| 23 | wire signed [32:0] mul_b = (funct3 == 3'b011 ¦¦ funct3 == 3'b010) |
| 24 | ? {1'b0, operand2} : {operand2[31], operand2}; |
| 25 | wire signed [65:0] full_product = mul_a * mul_b; |
| 26 | // MUL -> lower 32 bits; MULH/MULHU/MULHSU -> upper 32 bits |
| 27 | wire [31:0] mul_result = (funct3 == 3'b000) ? full_product[31:0] |
| 28 | : full_product[63:32]; |
| 29 |  |
| 30 | // ================================================================== |
| 31 | // 2. ITERATIVE DIVIDER FSM (32 cycles, restoring algorithm) |
| 32 | // ================================================================== |
| 33 | wire is_div = funct3[2]; // funct3[2]=1 => DIV/REM family |
| 34 |  |
| 35 | localparam S_IDLE = 2'b00, S_CALC = 2'b01, S_DONE = 2'b10; |
| 36 | reg [1:0] state = S_IDLE; |
| 37 | reg [4:0] counter = 5'd0; |
| 38 | reg [31:0] quotient = 32'd0; |
| 39 | reg [31:0] divisor = 32'd0; |
| 40 | reg [32:0] remainder = 33'd0; |
| 41 | reg invert_quotient = 1'b0; |
| 42 | reg invert_remainder = 1'b0; |
| 43 | reg is_rem = 1'b0; |
| 44 | reg div_by_zero = 1'b0; |
| 45 |  |
| 46 | always @(posedge clk or negedge reset_n) begin |
| 47 | if (!reset_n) begin |
| 48 | state <= S_IDLE; valid <= 1'b0; counter <= 5'd0; |
| 49 | quotient <= 32'd0; divisor <= 32'd0; remainder <= 33'd0; |
| 50 | invert_quotient <= 1'b0; invert_remainder <= 1'b0; |
| 51 | is_rem <= 1'b0; div_by_zero <= 1'b0; |
| 52 | end else begin |
| 53 | valid <= 1'b0; // default: de-asserted; pulses in S_DONE |
| 54 |  |
| 55 | case (state) |
| 56 | S_IDLE: begin |
| 57 | // Gate: !valid prevents FSM re-trigger on back-to-back |
| 58 | if (start && is_div && !valid) begin |
| 59 | is_rem <= funct3[1]; |
| 60 | div_by_zero <= (operand2 == 32'd0); |
| 61 | if (funct3 == 3'b100 ¦¦ funct3 == 3'b110) begin |
| 62 | // Signed: work with magnitudes, track signs |
| 63 | quotient <= operand1[31] ? (~operand1+1) : operand1; |
| 64 | divisor <= operand2[31] ? (~operand2+1) : operand2; |
| 65 | invert_quotient <= operand1[31] ^ operand2[31]; |
| 66 | invert_remainder <= operand1[31]; |
| 67 | end else begin // Unsigned |
| 68 | quotient <= operand1; |
| 69 | divisor <= operand2; |
| 70 | invert_quotient <= 1'b0; |
| 71 | invert_remainder <= 1'b0; |
| 72 | end |
| 73 | remainder <= 33'd0; |
| 74 | counter <= 5'd31; |
| 75 | state <= S_CALC; |
| 76 | end |
| 77 | end |
| 78 |  |
| 79 | S_CALC: begin |
| 80 | if ({remainder[31:0],quotient[counter]} >= {1'b0,divisor}) begin |
| 81 | remainder <= {remainder[31:0],quotient[counter]} - {1'b0,divisor}; |
| 82 | quotient[counter] <= 1'b1; |
| 83 | end else begin |
| 84 | remainder <= {remainder[31:0],quotient[counter]}; |
| 85 | quotient[counter] <= 1'b0; |
| 86 | end |
| 87 | if (counter == 5'd0) state <= S_DONE; |
| 88 | else counter <= counter - 1; |
| 89 | end |
| 90 |  |
| 91 | S_DONE: begin |
| 92 | if (div_by_zero) begin |
| 93 | quotient <= 32'hFFFF_FFFF; // RISC-V spec: DIV/0 = -1 |
| 94 | remainder <= {1'b0, operand1}; // REM/0 = dividend |
| 95 | end else begin |
| 96 | if (invert_quotient) quotient <= ~quotient + 1; |
| 97 | if (invert_remainder) remainder <= ~remainder + 1; |
| 98 | end |
| 99 | valid <= 1'b1; |
| 100 | state <= S_IDLE; |
| 101 | end |
| 102 | endcase |
| 103 | end |
| 104 | end |
| 105 |  |
| 106 | // ================================================================== |
| 107 | // 3. OUTPUT ROUTING |
| 108 | // ================================================================== |
| 109 | wire combinational_busy = (state == S_IDLE) && start && is_div && !valid; |
| 110 | assign busy = (state != S_IDLE) ¦¦ combinational_busy; |
| 111 | assign result = is_mul ? mul_result : (is_rem ? remainder[31:0] : quotient); |
| 112 |  |
| 113 | endmodule |

4.  Validation Using Simulation

4.1  Testbench Overview — tb_rv32m_alu.v

The testbench is a self-checking stress test that loads a 52-instruction hex program (imem_stress.hex) into the full pipeline DUT and checks all 29 register results after the program halts at PC = 0xCC. It covers seven test groups:

| Group | Category | Registers / Cases Checked |
| --- | --- | --- |
| A | Basic MUL / DIV / REM | x3–x5 : 15×5=75, 15÷5=3, 15%5=0 |
| B | Signed combinations | x6–x12 : all four sign permutations of (−7,3), (7,−3), (−7,−3) |
| C | Div-by-zero + MIN_INT/−1 | x13–x18 : RISC-V spec edge cases, both DIV and REM variants |
| D | Upper multiply (MULH/MULHU/MULHSU) | x19–x24 : verifies sign-extension of mul_a / mul_b |
| E | Unsigned division (DIVU/REMU) | x25–x26 : 0xFFFFFFFF ÷ 2 = 0x7FFFFFFF, remainder = 1 |
| F | Back-to-back MUL/DIV/REM | x27–x30 : 12×7, 12÷7, 12%7, then MUL again without intervening NOPs |
| G | Data-forwarding (DIV→MUL) | x31 : MUL uses quotient forwarded from the immediately preceding DIV |

4.2  Testbench Snippet

Clock/reset setup, halt detection, and the self-checking task.

Clock, Reset and Waveform

| 1 | // === Clock, Reset, Waveform Setup === |
| --- | --- |
| 2 | reg clk; |
| 3 | reg reset; |
| 4 |  |
| 5 | initial clk = 0; |
| 6 | always #5 clk = ~clk; // 100 MHz |
| 7 |  |
| 8 | initial begin |
| 9 | reset = 0; |
| 10 | #15; |
| 11 | reset = 1; |
| 12 | end |
| 13 |  |
| 14 | initial begin |
| 15 | $dumpfile("rv32m_alu.vcd"); |
| 16 | $dumpvars(0, tb_rv32m_alu); |
| 17 | end |

Halt Detection

| 1 | // === Halt Detection: PC loops at 0xCC when all instructions done === |
| --- | --- |
| 2 | reg [31:0] stuck_count; |
| 3 | localparam HALT_PC = 32'h000000CC; |
| 4 |  |
| 5 | always @(posedge clk) begin |
| 6 | if (!reset) stuck_count <= 0; |
| 7 | else begin |
| 8 | if (pc_out >= HALT_PC && !DUT.alu_busy) |
| 9 | stuck_count <= stuck_count + 1; |
| 10 | else if (pc_out < HALT_PC) |
| 11 | stuck_count <= 0; |
| 12 | end |
| 13 | end |

Self-Checking register comparisons (selected)

| 1 | // === Self-checking task: prints PASS/FAIL per register === |
| --- | --- |
| 2 | task check_reg; |
| 3 | input integer regnum; |
| 4 | input [31:0] expected; |
| 5 | reg [31:0] actual; |
| 6 | begin |
| 7 | actual = DUT.regs[regnum]; |
| 8 | if (actual === expected) |
| 9 | $display("[PASS] x%-2d = 0x%08H (%0d)", regnum, actual, $signed(actual)); |
| 10 | else |
| 11 | $display("[FAIL] x%-2d = 0x%08H expected 0x%08H (%0d != %0d)", |
| 12 | regnum, actual, expected, $signed(actual), $signed(expected)); |
| 13 | end |
| 14 | endtask |
| 15 |  |
| 16 | // Groups A-G covering all 29 test cases: |
| 17 | // Group A: Basic MUL/DIV/REM |
| 18 | check_reg( 3, 32'h0000004B); // 75 = 15*5 |
| 19 | check_reg( 4, 32'h00000003); // 3 = 15/5 |
| 20 | check_reg( 5, 32'h00000000); // 0 = 15%5 |
| 21 | // Group B: Signed combinations |
| 22 | check_reg( 6, 32'hFFFFFFEB); // -21 = (-7)*3 |
| 23 | check_reg( 7, 32'hFFFFFFFE); // -2 = (-7)/3 |
| 24 | check_reg( 8, 32'hFFFFFFFF); // -1 = (-7)%3 |
| 25 | // Group C: Div-by-zero + signed overflow (MIN_INT / -1) |
| 26 | check_reg(13, 32'hFFFFFFFF); // DIV/0 -> -1 |
| 27 | check_reg(14, 32'h0000002A); // REM/0 -> dividend |
| 28 | check_reg(17, 32'h80000000); // MIN_INT/-1 -> MIN_INT |
| 29 | check_reg(18, 32'h00000000); // MIN_INT%-1 -> 0 |
| 30 | // Group D: Upper multiply (MULH/MULHU/MULHSU) |
| 31 | check_reg(19, 32'h00000001); // MULH (0x40000000, 4) = 1 |
| 32 | check_reg(23, 32'hFFFFFFFE); // MULHU (0xFFFFFFFF, 0xFFFFFFFF) |
| 33 | // Group E: Unsigned division, Group F: Back-to-back, Group G: Forwarding |
| 34 | check_reg(25, 32'h7FFFFFFF); // DIVU(0xFFFFFFFF, 2) |
| 35 | check_reg(31, 32'h00000054); // 84 = 1*84 (forwarded result) |

4.3  Icarus Verilog Simulation Results

Simulation command and full terminal output — 29/29 tests pass. The annotated line shows the pipeline freezing for exactly 32 clock edges (320 ns @ 100 MHz) while the divider counts down to zero.

$ iverilog -o sim_rv32m.out -I ./rtl ./rtl/*.v ./tb/tb_rv32m_alu.v

$ vvp sim_rv32m.out

t=25 PC=0x0000 busy=0 stall=0

t=35 PC=0x0004 busy=0 stall=0

t=45 PC=0x0008 busy=0 stall=0

t=55 PC=0x000C busy=0 stall=0

t=75 PC=0x0014 busy=1 stall=1 ← DIV enters S_CALC, pipeline frozen

t=395 PC=0x0014 busy=0 stall=0 ← 32 cycles elapsed, S_DONE fires

...

================================================================

Program halted at PC = 0x00CC after 3240 ns

Checking all register values...

================================================================

--- Group A: Basic MUL/DIV/REM ---

[PASS] x3 = 0x0000004B (75)

[PASS] x4 = 0x00000003 (3)

[PASS] x5 = 0x00000000 (0)

--- Group B: Signed combinations ---

[PASS] x6 = 0xFFFFFFEB (-21)

[PASS] x7 = 0xFFFFFFFE (-2)

[PASS] x8 = 0xFFFFFFFF (-1)

[PASS] x9 = 0xFFFFFFFE (-2)

[PASS] x10 = 0x00000001 (1)

[PASS] x11 = 0x00000002 (2)

[PASS] x12 = 0xFFFFFFFF (-1)

--- Group C: Division by zero + signed overflow ---

[PASS] x13 = 0xFFFFFFFF (-1)

[PASS] x14 = 0x0000002A (42)

[PASS] x15 = 0xFFFFFFFF (4294967295)

[PASS] x16 = 0x0000002A (42)

[PASS] x17 = 0x80000000 (-2147483648)

[PASS] x18 = 0x00000000 (0)

--- Group D: MULH / MULHU / MULHSU ---

[PASS] x19 = 0x00000001 (1)

[PASS] x20 = 0x00000001 (1)

[PASS] x21 = 0x00000001 (1)

[PASS] x22 = 0x00000000 (0)

[PASS] x23 = 0xFFFFFFFE (-2)

[PASS] x24 = 0xFFFFFFFF (-1)

--- Group E: Unsigned division ---

[PASS] x25 = 0x7FFFFFFF (2147483647)

[PASS] x26 = 0x00000001 (1)

--- Group F: Back-to-back MUL/DIV/REM ---

[PASS] x27 = 0x00000054 (84)

[PASS] x28 = 0x00000001 (1)

[PASS] x29 = 0x00000005 (5)

[PASS] x30 = 0x00000054 (84)

--- Group G: Data dependency / forwarding ---

[PASS] x31 = 0x00000054 (84)

================================================================

RESULTS: 29 passed, 0 failed out of 29 tests

>>> ALL TESTS PASSED <<<

================================================================

4.4  GTKWave Waveform

[ TODO: Paste GTKWave screenshot here — open rv32m_alu.vcd, add signals listed in Section 2, zoom to show at least one full DIV operation (32 S_CALC cycles) and the resulting valid pulse ]

GTKWave file generated by: vvp sim_rv32m.out → rv32m_alu.vcd. Open with gtkwave rv32m_alu.vcd & from the hardware/cpu/ directory.

| Signal | Expected behaviour in waveform |
| --- | --- |
| clk, reset_n | clk, reset_n |
| start, funct3[2:0] | start, funct3[2:0] |
| operand1[31:0], operand2[31:0] | operand1[31:0], operand2[31:0] |
| state[1:0] | 2'b00=IDLE, 2'b01=CALC, 2'b10=DONE) |
| counter[4:0] | counts from 31 down to 0 during S_CALC) |
| busy | HIGH throughout S_CALC; pipeline stalls during this window) |
| valid | single-cycle pulse in S_DONE) |
| result[31:0] | valid 1 cycle after valid pulse) |

5.  FPGA Demo

| Item | Status / Notes |
| --- | --- |
| Module functionality checked on FPGA | Partial — rv32m_alu synthesises cleanly in Vivado (Artix-7 XC7A100T). DSP48E1 inference confirmed in utilisation report (4 DSP blocks used). Full board-level verification pending bitstream upload. |
| Synthesis result | 13,184 LUT / 4,111 LUTRAM / 5,431 FF / 5 BRAM / 4 DSP — within device budget (20.79% LUT utilisation). |
| Demo 1 status | Demo 1 ran a simplified pipeline subset (no coprocessor). The rv32m_alu module was present in the synthesis but full M-extension stress was deferred to the final demo. |

6.  Learnings

| Theme | Detail |
| --- | --- |
| Challenge | The most painful bug was the FSM re-trigger loop. Without gating start on !valid in S_IDLE, the combinational busy signal held start high into the next cycle (pipeline was stalled, so the same instruction kept re-presenting), causing S_CALC to restart every time it reached S_DONE. Division never completed. Adding !valid as a guard resolved it immediately. |
| Insight | Using plain * on correctly sign-extended wires is enough for Vivado to infer DSP48E1 blocks — no vendor primitives required. The same technique is reused in conv_datapath.v for the convolution MAC array, making inference consistent across the whole design. |